The Enterprise has a 22-bit address space (A0 to A21) giving 4MB of addressable space!

The Z80 address space is divided into 16KB “Pages” (numbered 0-3) where A14 and A15 select four 8-bit “Segment” registers in “DAVE”.

The Enterprise 4MB address space is divided into 256 16K “segments”. Each “Segment” has a number, 00h-FFh.

A0-A13 + 8-bits, from the “Segment” registers, generate the 22-bit address.

Certain “Segments” are predefined

| Internal 32K ROM | 00h-01h |
| --- | --- |
| 64K Cartridge Slot | 04h-07h |
| Internal 64K DRAM | FCh-FFh |
| 2nd Internal 64K DRAM | F8h-FBh |

There is Z80 I/O port associated with each “Segment” register in “DAVE” which allows the “Segment” number to be set for each “page”.

| Page | Address | Port |
| --- | --- | --- |
| 0 | 0000h-3FFFh | B0h |
| 1 | 4000h-7FFFh | B1h |
| 2 | 8000h-BFFFh | B2h |
| 3 | C000h-FFFFh | B3h |

Any “Segment” can be mapped to any “Page” by writing the “Segment” number to the IO port for the respective “Page”.

Using 512KB SRAM (AS6C4008) with 19 address lines (A0 to A18), the “Segments” are as below;

| Segment | A21 | A20 | A19 | A18 | A17 | A16 | A15 | A14 | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FF | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | Internal DRAM |
| FE | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | Internal DRAM |
| FD | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | Internal DRAM |
| FC | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | Internal DRAM |
| FB | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 512K SRAM |
| FA | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 512K SRAM |
| F9 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 512K SRAM |
| F8 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 512K SRAM |
| F7 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 512K SRAM |
| F6 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 512K SRAM |
| F5 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 512K SRAM |
| F4 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 512K SRAM |
| F3 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 512K SRAM |
| F2 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 512K SRAM |
| F1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 512K SRAM |
| F0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 512K SRAM |
| EF | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 512K SRAM |
| EE | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 512K SRAM |
| ED | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 512K SRAM |
| EC | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 512K SRAM |
| EB | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 512K SRAM |
| EA | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 512K SRAM |
| E9 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 512K SRAM |
| E8 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 512K SRAM |
| E7 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 512K SRAM |
| E6 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 512K SRAM |
| E5 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 512K SRAM |
| E4 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 512K SRAM |
| E3 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 512K SRAM |
| E2 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 512K SRAM |
| E1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 512K SRAM |
| E0 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 512K SRAM |
| DF | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 512K SRAM |
| DE | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 512K SRAM |
| DD | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 512K SRAM |
| DC | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 512K SRAM |
| DB | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | No SRAM |

Note DF-DC maps over the Internal DRAM “Segments” but with A19 low.

Decoded Address Line signals to drive CE on the 512KB SRAM are as follows;

(A21 & A20 & A19 & A18 & A17 & !A16)

# (A21 & A20 & A19 & A18 & !A17)

# (A21 & A20 & A19 & !A18)

# (A21 & A20 & !A19 & A18 & A17 & A16)

Additionally, the Control Lines for a Memory Request and excluding Refresh cycles are required.

!MREQ & REFSH

ChatGPT reduced this but it is easier to follow the  above.

!CE=A21 & A20 & !MREQ & RFSH & (A19 & !A18 # A19 & /A17 # A19 & /A16 # /A19 &A18 & A17 &A16)